National Textile University, Faisalabad

Department of Computer Science

| Name | Sarwat Fatima |
| --- | --- |
| Class | BSCS – 5th – A |
| Section | 5th -A |
| Registration No | 23-NTU-CS-1092 |
| Course Name | Embedded Iot systems |
| Submitted To | Sir Nasir Mehmod |
| Submission Date | 22/12/2025 |

Smart Classroom Project Documentation

This project implements a Smart Classroom monitoring system using an ESP32, PIR and LDR sensors, Blynk IoT platform, and a custom web dashboard. It enables real-time tracking of student presence, presentation engagement, and performance feedback in a classroom environment, helping teachers to evaluate and record student activity efficiently.

# Overview

The code combines sensor data, Blynk virtual widgets, and a built-in web server to deliver a comprehensive view of classroom dynamics. It connects to WiFi, reads sensor inputs to gauge student activity and engagement, updates a mobile Blynk dashboard, and hosts a web page with analytics and feedback for each student.

# Key Features

Student Activity Monitoring: Detects presence and presentation activity using PIR and LDR sensors.

Blynk Integration: Visualizes and controls session data via a mobile app.

Feedback & Analytics: Generates feedback and engagement scores per student.

Web Dashboard: Custom web page showing real-time stats, feedback, and charts.

# Main Components & Architecture

ESP32

:

Executes the main logic and hosts the web server.

PIR Sensor

:

Detects motion (student presence

).

LDR Sensor

:

Detects light changes (projector state

).

Blynk IoT

:

Provides virtual widgets and mobile UI.

WebServer

:

Hosts a dashboard with a table, analytics, and Chart.js visualizations.

## Project Structure

Section

Purpose

WiFi Setup

Connects ESP32 to local WiFi

Sensor Reading

Reads PIR and LDR, calculates presence and presentation

Student Management

Selects and updates current student info

Feedback Generation

Provides automated feedback based on performance

Web Dashboard

Displays real-time data and analytics

Blynk Callbacks

Handles UI interactions and data saving

# Global Variables and Configuration

#define BLYNK_TEMPLATE_ID "TMPL6bnrG4pnK"

#define BLYNK_TEMPLATE_NAME "Smart ClassRoom"

#define BLYNK_AUTH_TOKEN "S6ratnohzoGe9T94WaLasaZ2I6pIge4s"

#define BLYNK_PRINT Serial

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

#include <WebServer.h>

char ssid[] = "Haru";

char pass[] = "Haruki12";

// ================== PINS ==================

const int pirPin = 34;

const int ldrPin = 35;

// ================== VIRTUAL PINS ==================

#define V_SELECT_STUD   V2

#define V_ACTIVE_STUD   V3

#define V_STUDENT_PIR   V5

#define V_PROJECTOR     V6

#define V_PRES_TIME     V10

#define V_ATTENDANCE    V12

#define V_SCORE         V15

#define V_REMARKS       V16

#define V_SAVE          V17

#define V_SAVED_SCORE   V18

#define V_SAVED_REMARKS V19

WebServer server(80);

BlynkTimer timer;

// ================== STUDENT DATA ==================

struct StudentData {

String name;

int presPercent;        // Presentation timing %

int presencePercent;    // Attendance %

int score;

String remarks;

};

StudentData students[] = {

{"Haruki", 0, 0, 0, ""},

{"Mano", 0, 0, 0, ""},

{"Ibana", 0, 0, 0, ""},

{"Amna", 0, 0, 0, ""}

};

# Sensor Reading and Engagement Calculation

The system continuously reads inputs from the PIR and LDR sensors:

PIR Sensor: Detects student motion, marking them present.

LDR Sensor: Detects when the projector is ON (increased light level).

Presence Percent: Calculated by time spent detected by PIR.

Presentation Percent: Calculated by time projector is ON.

## Engagement Calculation Algorithm:

int calculateEngagement(StudentData s){

int baseScore = (s.presencePercent * 0.4) + (s.presPercent * 0.6);

if(s.remarks.indexOf("participat") >= 0) baseScore += 10;

if(s.presencePercent < 50) baseScore -= 15;

if(baseScore < 0) baseScore = 0;

if(baseScore > 100) baseScore = 100;

return baseScore;

}

Weighting: 40% attendance, 60% presentation.

Bonus: +10 if remarks mention 'participation'.

Penalty: -15 if presence < 50%.

Clamped: 0–100%.

Sensor Data Update Flow

presence info

projector info

readSensors called every 1s

PIR Sensor - Motion

LDR Sensor - Projector State

Compute presence/presentation %

Update Blynk Widgets

Update Student Data

# Blynk Virtual Pins

Virtual Pin

Purpose

V2

Student selection dropdown

V3

Displays current student name

V5

PIR sensor status

V6

Projector state

V10

Presentation time %

V12

Attendance %

V15

Score input

V16

Remarks input

V17

Save button

V18

Saved score display

V19

Saved remarks display

# Student Selection, Scoring, and Saving

Score/Remarks: Updates student struct fields.

Save: Stores results and ends session.

# Feedback Generation

Feedback is dynamically generated for each student based on their attendance, presentation, and remarks:

String generateFeedback(StudentData s) {

String feedback = "<ul class='feedback-list'>";

// Attendance feedback

if(s.presencePercent >= 15) {

feedback += "<li> Excellent class participation!</li>";

} else if(s.presencePercent >= 10) {

feedback += "<li> Good engagement, try to participate more</li>";

} else {

feedback += "<li> Needs to increase participation</li>";

}

// Presentation feedback

if(s.presPercent >= 15) {

feedback += "<li> Outstanding presentation skills</li>";

} else if(s.presPercent >= 10) {

feedback += "<li> Solid presentation delivery</li>";

} else if(s.presPercent > 0) {

feedback += "<li> Practice presentation timing</li>";

}

// Custom remarks

if(s.remarks.length() > 0) {

feedback += "<li> Teacher note: " + s.remarks + "</li>";

}

feedback += "</ul>";

return feedback;

# Web Dashboard

The ESP32 hosts a web server at its local IP, accessible over LAN.

## Web Page Features

Student Overview Table: Displays all students with attendance, presentation, scores, remarks, and feedback.

Color Coding: Attendance and presentation cells color-coded for quick status.

Teacher Notes: Highlights the top and lowest engaged students.

Chart.js Visualization: Bar chart compares engagement scores.

Web Dashboard Layout

requests

retrieves

generates HTML

sent to

renders

Web Client Browser

ESP32 WebServer

Student Data Array

Feedback Generator

Chart.js

# Code Flow Summary

- Setup: Initializes pins, Blynk, WiFi, timers, and web server.

- Loop: Handles Blynk UI, sensor timer, and web server requests.

- Sensor Timer: Reads sensors, updates student/session data, pushes to Blynk.

- Blynk Handlers: Receive UI changes (student, score, remarks, save).

- WebServer Handler: Generates styled HTML dashboard with live data and chart.

# Setup and Deployment

- Connect Sensors: Attach PIR to pin 34, LDR to pin 35.

- Configure WiFi: Update ssid and pass in the code.

- PlatformIO: Upload firmware to ESP32 via VSCode.

- Blynk: Use credentials and set up widgets as specified.

- Access Web Dashboard: Open http://<ESP32-IP> in browser on same network.

# Conclusion

This Smart Classroom project provides an interactive, sensor-driven evaluation system for teachers, combining mobile and web dashboards for optimal accessibility. The code can be customized for different environments by adjusting student data, sensors, and UI.

ScreenShots: